به نام خدا

عنوان:

تکلیف شماره دوم درس طراحی سیستم‌های دیجیتالی

نام درس:

طراحی سیستم‌های دیجیتالی

استاد درس:

دکتر مروستی

اعضا:

محمدعلی مجتهدسلیمانی-992023039

تاریخ:

20/02/1403

Q1. For verify a 4-bit carry look-ahead adder we should first make a test bench to simulate all possible inputs and check the outputs against expected values which test bench going to do that. For the following code I assumed that you already have a Carry-Look-Ahead Adder module, for my code I already done it in first homework question 4.

- VHDL Test Bench Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity TestBench_CLA is

end TestBench_CLA;

architecture behavior of TestBench_CLA is

component Carry_Look_Ahead_Adder

Port (

A : in STD_LOGIC_VECTOR(3 downto 0);

B : in STD_LOGIC_VECTOR(3 downto 0);

Cin : in STD_LOGIC;

Sum : out STD_LOGIC_VECTOR(3 downto 0);

Cout : out STD_LOGIC

);

end component;

-- Test inputs

signal A, B: STD_LOGIC_VECTOR(3 downto 0);

signal Cin: STD_LOGIC;

signal Sum: STD_LOGIC_VECTOR(3 downto 0);

signal Cout: STD_LOGIC;

begin

-- Instantiate the Unit Under Test (UUT)

uut: Carry_Look_Ahead_Adder port map (

A => A,

B => B,

Cin => Cin,

Sum => Sum,

Cout => Cout

);

-- Stimulus process to apply test vectors

stimulus: process

begin

-- Apply a set of test vectors

A <= "0000"; B <= "0000"; Cin <= '0'; wait for 10 ns;

A <= "0001"; B <= "0011"; Cin <= '0'; wait for 10 ns;

A <= "0101"; B <= "0101"; Cin <= '1'; wait for 10 ns;

A <= "1111"; B <= "1111"; Cin <= '1'; wait for 10 ns;

-- Complete the testing

-- wait, then end the process

wait;

end process;

end behavior;

- Complication Report:

- RTL view:

- Post-mapping view:

Q2. For a T flip flop, we the next state of flip flop driven by XOR T (input of circuit) and Q which is the current state of our circuit so based on this and based on t ff truth table I wrote a code for it. First here is a truth table of t ff:

- T FF truth table:

- Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tff is

Port ( clk : in STD_LOGIC;

async_reset : in STD_LOGIC;

Q : out STD_LOGIC);

end entity tff;

architecture toggle of tff is

signal S_Q : std_LOGIC := '0';

begin

process(clk, async_reset)

begin

if async_reset = '1' then

S_Q <= '0';

elsif rising_edge(clk) then

S_Q <= not S_Q;

end if;

end process;

Q <= S_Q;

end architecture toggle;

*: in this code I declared 2 inputs: clk for clock pin of circuit and the async_reset for reset pin of circuit. The output is Q which is next state of circuit. Reset pin Is asynchronous from clock pin and it is active high so it means when pin becomes 1 the output of circuit be 0. I used signals to work with outputs Q because we can not directly work with Q. signal is a wired in VHDL.

- Output after compile:

- RTL view:

*: notice that in this code we built a T flip flop using D flip flop. This is achieved by feeding the output of the D Flip flop to its input through an XOR gate.

- Post-mapping view:

- Wave Form:

- Test bench code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_tff is

-- Testbench does not have any ports

end tb_tff;

architecture behavior of tb_tff is

-- Component Declaration for the T Flip-Flop

component tff

Port ( clk : in STD_LOGIC;

async_reset : in STD_LOGIC;

Q : out STD_LOGIC);

end component;

-- Signals for interfacing with the component

signal tb_clk : STD_LOGIC := '0';

signal tb_reset : STD_LOGIC := '0';

signal tb_Q : STD_LOGIC;

-- Clock period definition

constant clk_period : time := 10 ns;

begin

-- Instantiate the T Flip-Flop

uut: tff port map (

clk => tb_clk,

async_reset => tb_reset,

Q => tb_Q

);

-- Clock process definitions

clk_process : process

begin

while true loop

tb_clk <= '0';

wait for clk_period/2;

tb_clk <= '1';

wait for clk_period/2;

end loop;

end process;

-- Stimulus process

stim_proc: process

begin

-- Test 1: Reset the flip-flop

tb_reset <= '1';

wait for 25 ns;

tb_reset <= '0';

wait for 25 ns;

-- Test 2: Allow some clock pulses to toggle the flip-flop

tb_reset <= '0';

wait for 100 ns;

-- Test 3: Assert reset while clock is running

tb_reset <= '1';

wait for 20 ns;

tb_reset <= '0';

wait for 100 ns;

-- End of testing

wait;

end process;

end behavior;

Q3. Designing a 4-bit BCD (Binary-Coded Decimal) up/down counter in VHDL involves creating a circuit that counts from 0 to 9 and back again, with the count direction controlled by an input signal Dir.

- Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity BCD_Counter is

Port (

clk : in STD_LOGIC;

reset : in STD_LOGIC;

Dir : in STD_LOGIC;  -- '0' for down, '1' for up

count : out STD_LOGIC_VECTOR(3 downto 0)

);

end BCD_Counter;

architecture Behavioral of BCD_Counter is

signal r_count : STD_LOGIC_VECTOR(3 downto 0) := "0000";

begin

process(clk, reset)

begin

if reset = '1' then

r_count <= "0000";  -- Reset the counter

elsif rising_edge(clk) then

if Dir = '1' then  -- Count up

if r_count = "1001" then  -- Max value for BCD

r_count <= "0000";  -- Wrap around

else

r_count <= r_count + 1;

end if;

else  -- Count down

if r_count = "0000" then

r_count <= "1001";  -- Wrap around from 0 to 9

else

r_count <= r_count - 1;

end if;

end if;

end if;

end process;

count <= r_count;  -- Output the current count

end Behavioral;

compilation report:

RTL view:

Post-mapping view: